Chemical Engineering 150A Final Review Sheet

Fluid statics

|  |  | dPdz=-ρg P2-P1=ρgz2-z1 P=ρgh+P0 2 points connected in same fluid at the same height, pressure is equal |  |  |
| --- | --- | --- | --- | --- |
|  |  | W+ PatmA= PinA PinVfinal=PatmVinitial Vfinal=Vinitial-Ah PinVinitial-Ah=PatmVinitial |

Compressible fluid

Momentum Balance

Straight tube:		F= ρbub2Sb-ρaua2Sa+PbSb-PaSa

U tube:		F= ρbub2Sb+ρaua2Sa+PbSb+PaSa

Velocity entering the fitting- push the fitting in the direction of velocity

Velocity leaving the fitting- push the fitting in the opposite direction of velocity

Energy Balance

| Turbulent flow |  |  |  | W=∆Pρ+∆u22+g∆z+ hf |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Laminar flow |  |  |  | W=∆Pρ+∆u2+g∆z+ hf |  |  |
| Terms | + |  |  |  |  | - |
| W | Work done on the system |  |  |  |  | Work done by the system |
| ∆Pρ | Inlet > outlet, start with high pressure |  |  |  |  | Inlet < outlet, end with high pressure |
| ∆u2 | Start with high velocity, decelerating |  |  |  |  | Turn into high velocity, accelerating |
| g∆z | Start at higher point, going into low point |  |  |  |  | Start with low point, elevation is raised |
| hf | Always positive, work is dissipated as friction hf=4f (LD)v22 |  |  |  |  |  |
|  |  |  | Laminar |  | Turbulent |  |
| Newtonian Re=DρVμ Recritical=2100 |  |  | f=16Re |  | Churchill’s correlation (more accurate) f=2[8Re12+1(2.457ln17Re0.9+0.27kD16+37530Re16)32]112 Colebrook equation (easier) 1f=-4log⁡[k/D3.7+1.255Ref] |  |
| Power law fluid |  |  |  |  |  |  |  |  |
|  |  |  | Re=23-nn3n+1nv2-nDnρK Recritical=21004n+25n+333n+12 |  |  |  |  |  |
| Laminar |  |  | ∆P=2n+23n+1nnLKvDn+1 f=16Re=2n+13n+1nnKv2-nDnρ |  |  |  |  |  |
| Turbulent |  |  |  |  |  |  |  |  |

L is the length of the tube

D is the diameter of the tube

µ is the viscosity of the fluid

k is the roughness factor

| Bingham Plastic | Re=Dρvμ∞ He=D2ρτ0μ∞2 |
| --- | --- |
| Laminar | f=16Re1+He6Re-He43f3Re7 |
| Turbulent | f=10aRe-0.193 a=-1.378(1+0.146e-2.9*10-5He) |

Flow through non-circular conduits			Deq=4RH=4SLp

RH =  hydraulic radius			S =  cross-sectional area		Lp =  wetted perimeter

Bernoulli

| hf=4flD+Kfv22+∆Punitρ Total friction = Skin friction + Fitting drag + Unit drag Ke = 1 for filling the tank Kc = 0.4 for emptying the tank Pumps Cavitation occurs when Pinlet < Pvap. To pump efficiently, Pinlet > Pvap NPSH+Pvapρ≤Pρ+v22inlet =(Pρ+v22+gz-zinlet-hf)upstream |  |
| --- | --- |

Flow around immersed objects

Particles reach terminal velocity when  F=Fg+Fk+Fg+FB=0

Fg=mpg=Vpρpg             Fk=CdAρu022             FB=AP2-P1=Aρfgh=Vpρf

ut=4gρp-ρpDp3Cdρf

Cd is the drag coefficient; A is the projected area of the particle

Re=Du0ρμ

| Stoke | Newton |
| --- | --- |
| Re<1 | 1000<Re<200000 |
| Fk=3πμDu0 | Fk=0.055πμD2u02 |
| Cd=24Re | Cd≅0.44 |
| ut=Dp2ρp-ρf18μ | ut=1.75gDpρp-ρfρf |
| K=gρfρp-ρfμ213 K < 2.6 | K > 68.9 |

Fixed and fluidized bed

∆PErgun=150μv0Lb∅s2Dp21-ε2 ε3+1.75ρv02Lb∅sDb1-εε3

hf=∆PErgunρ      Lb:bed length         Dp: equivalent spherical particle diameter

Dp=6Vparticleπ13        ∅s=asphereaparticle=6DpSparticleVparticle      ε=1-ρbulkρp

Minimum fluidization velocity

ρp-ρfg=ρfvmf2∅sDpεmf31501-εmfμ∅sDpρfvmf+1.75        ∅sεmf3≅114              Lb,mf=MpAb1-εmfρp

Compressible flow

| Isothermal flow | Isentropic flow |
| --- | --- |
| usound=zRTM | usound=γRTM |
| If flow is subsonic (normal), then pressure will drop from Pinlet to Pexit in pipe G2=MRTP12-P224fLD-lnP2P12 Check that u2 (end of pipe) < usound to make sure flow is subsonic If flow goes sonic (choked), pressure will not be able to drop to Pexit at the end Gmax2=ρ1P11+4fLD+ln⁡(ρ1P1Gmax2) Check sonic flow [P2 (end of pipe) > Pexit]: Gmax2=ρ2P2 | With enough ∆P a fluid can go sonic at the throat, and supersonic in diverging nozzle. N=vvsound γ=CPCV AA*=1Ma2+γ-1N2γ+1γ+12γ-1 TT0=1+γ-12N2-1 PP0=1+γ-12N2γ1-γ ρρ0=1+γ-12N211-γ The maximum velocity at the throat is NMa = 1, there is a maximum flow rate mmax=A*γρ0P02γ+1γ+1γ-1 |

|  | Due to imperfections in nozzle, flow can drop off the isentropic pathway as it decelerates to v < vsound. Normal shock tables relate conditions before and after a shock. - Find Ma on isentropic table for the given shock location - Look up Ma as Ma1 on normal shock table, record ratios of post shock values Ma2, P2, and T2 relative to P1 and T1 before shock. After normal shock, fluid will continue to travel on a new isentropic pathway - Use isentropic flow table to find fictional A*, T0, and P0 that would have given you the same conditions as the post-chock conditions. - Use isentropic flow table to find conditions further down the nozzle relative to those A*, T0, and P0 values |
| --- | --- |

Compressors

Compressor ratio (C.R.)=PdischargePsuction

Determine number of required centrifugal compressor stages:

o Guideline (1) max compression ratio/stage ~3

o Guideline (2) max discharge temperature ~350°F

C.R.total =C.R.stagen

Work per stage= W=P1γρ1γ-1P2P1γ-1γ-1

P=nWmη

Method for using chart:

1) Find starting point using initial P and T

2) Read off H at that point

3) Follow a line of constant S from starting point to ending P (of that stage)

4) Read off T and H at that end point

5)  ∆Hideal=Hend-Hend       ∆Hactual=∆Hactualη

6) Repeat for multiple stages, assuming perfect intercooling

Calculate final temperature

T2=T1P2P1γ-1γ

Wη=CP∆T

Chart: start at Pend, Tend, go right by ∆Hactual-∆Hideal to get T2, actual

Polytropic compression

Work per stage= W=P1nρ1n-1P2P1n-1n-1

P=jWmη    j is the number of stages

n=ηγ1+ηγ-γ

Navier-Stokes

ρ∂vx∂t+vx∂vx∂x+vy∂vx∂y+vz∂vx∂z=-∂p∂x+μ∂2vx∂x2+∂2vx∂y2+∂2vx∂z2+ρgx

ρ∂vy∂t+vx∂vy∂x+vy∂vy∂y+vz∂vy∂z=-∂p∂y+μ∂2vy∂x2+∂2vy∂y2+∂2vy∂z2+ρgy

ρ∂vz∂t+vx∂vz∂x+vy∂vz∂y+vz∂vz∂z=-∂p∂z+μ∂2vz∂x2+∂2vz∂y2+∂2vz∂z2+ρgz

Equation of continuity

∂ρ∂t+∂(ρvx)∂x+∂(ρvy)∂y+∂(ρvz)∂z=0

|  |  | Equation of continuity |
| --- | --- | --- |
| Assumptions | Implications |  |  |
| Flow is steady, d/dt = 0 | ∂vx∂t, ∂vy∂t, ∂vz∂t,∂vr∂t,∂v∅∂t=0 |  |  |
| Plates are infinite in - x/y direction - x/z direction - y/z direction Parallel flow | - vz=0 - vy=0 - vx=0 |  |  |
| Incompressible, Newtonian, laminar, constant properties |  |  |  |
| No pressure gradient | ∂P∂x or∂P∂y or ∂P∂z=0 |  |  |
| 2D | - vy=0, ∂vy∂y=0, ∂2vy∂y2=0 - vz=0, ∂vz∂z=0, ∂2vz∂z2=0 - vz=0, ∂vz∂z=0, ∂2vz∂z2=0 |  |  |
| Gravity acts in the –x,-y, or -z direction | gx or gy or gz= -g The other two g components are therefore 0 |  |  |
| No slip at wall | vx=vy=vz=0 |  |  |
| At the free surface (x = h), there is negligible shear | - ∂vy∂x=0 - ∂vz∂x=0 - ∂vz∂y=0 |  |  |
| The pipe is infinitely long in the z-direction, parallel flow | ur=0 ∂(rur)∂r=0 |  |  |
| The velocity field is axisymmetric with no swirl | u = 0 and all partial derivatives with respect to are zero |  |  |

Conduction

k – thermal conductivity, in units of power per distance per temperature (Btu / (hr ft F)

α – thermal diffusivity, in units of area per time (ft^2 / hour)

Thermal Resistance

q=∆TR

Plane: R=∆xkA           Cylinder: R=∆rkALM      Sphere: R=∆rkAGM             AGM=A1A2              ALM=A2-A1lnA2A1

Series resistance: Rtotal=R         Parallel resistance: 1Rtotal=1R

Non-steady state conduction

α=kρCp                  Fa=αtTr2 (unitless)

Ts – constant average temperature of surface

Tb – average temperature of the slab/sphere at time t or at position r

Ta – initial temperature of the slab/sphere

Figure 10.5 Average temperatures during unsteady-state heating or cooling of a large slab, and infinitely long cylinder, or a sphere.

|  | For infinitely long (no end effects) cylinder: θ=Ts-TbTs-Ta =0.692e-5.78Fa+0.131e-30.5Fa+0.0534e-74.9Fa For a sphere: θ=Ts-TbTs-Ta =0.608e-9.87Fa+0.152e-39.5Fa+0.0676e-88.8Fa |
| --- | --- |

Biot number: Bi=convectionconduction=hsk for a flat plate, where s is half of the thickness

Bi=hrk, for a sphere, where r is the radius

For constant surface

To get time, based on all temperatures

Get θ  Fig 10.5 to Get F0  Get time

To get average temperature, based on time

Based on time, get F0  Fig 10.5 t go get θ

For convective film

To get time, based on all temperatures

Calculate Re and Pr  Get Nu  Get h  Get Bi && Get θ  Fig 10.7/10.8 to Get F0  Get time

To get average temperature, based on time

Calculate Re and Pr  Get Nu  Get h  Get Bi && Get F0  Fig 10.7/10.8 to get θ based on F0 and Bi

Figure 10.7 Change with time of the average temperature of a slab with external convective resistance.

Convection

q=hA∆T

h is the individual heat transfer coefficient, obtained from empirical methods, power per area per temperature

NNu=hDk=0.023NRe0.8NPr0.333∅0.14=0.023Dρvμ0.8Cpμk0.333μμw0.14

For flow in pipes, 0.6<NPr<16700

NStNPr2/3∅0.14=hCpρvCpμk2/3μμw0.14=0.023Dρvμ-0.2

j=0.023Dρvμ-0.2=f2

|  | External forced convection normal to tubes NPr0.333NNu=0.35+0.56NRe0.52 |
| --- | --- |
|  | Flow past single sphere NNu=2.0+0.6NRe0.50NPr0.333 |

1U0A0=1h0A0+∆rinsulAlmkinsul+∆rpipeAlmkpipe+1hiAi

1U0=D0Dihi+D0ln⁡(D0/Di)2kw+1h0=D0Dihi+D0(D0-Di)DLM2kw+1h0

Where kw is the thermal conductivity of the pipe wall and D0-Di2 is the wall thickness xw

1Ui=1hi+Diln⁡(D0/Di)2kw+DiD0h0

Concentric pipe

Q=UA∆TLM

If ∅1=∅2, then ∆TLM=∅1

Z=Tha-ThbTcb-Tca        η=Tca-TcaTha-Tca

h- shell

c- tube

a- in

b- out

Q = U0A0FG∆TLM

Radiation

Kirchoff’s law of thermal radiation    ε1=α1 at thermal equilibrium

ε1=1 for blackbody

Energy input εσATs4   		Energy output εσAT∞4          q=εσA(Ts4-T∞4)

q1→2=σA1F12(T14-T24)       q2→1=σA2F21(T24-T14)

Conversion

1 lb=453.6 g

1 in=2.54 cm

1 ft3=7.48 gallon=28.32 L=0.02832 m3

1 ft=0.3048 m=30.48 cm       1 m=3.28 ft

1 atm=1013250 dyncm2=14.696 psi

1 dyn=10-5N= 10-5kg*ms2=1 g*cms2=2.248*10-6lbf

1 lbf=1 lbm*32fts2

1 P= 0.100 kgm*s=1 gcm*s= 0.06720lbft*s

1 cP= 0.01 P=0.0006720lbft*s

1=32lbm*ftlbf*s2

oR= oF+459.7

R=8.3145Jmol*K=1545.3ft*lbf oR lb mol

ρH2O=1 gcm3=62.4 lbmft3

1 Hp=745.7 W=550 ft*lbfs

g=9.8 ms2=32.17 fts2

1 Btu=1055.05 J           1Btus=1055.05 W=1.41485 Hp

1 Hp=745.7 W=550 ft*lbfs